| Safe System of Work |  |
| --- | --- |

| Company name | Elite Services Holdings Ltd | Department | Service |
| --- | --- | --- | --- |

| Contract | Customers Site | Prepared By | Gareth Lewis |
| --- | --- | --- | --- |

| Date prepared | 01/05/2024 | Review date | 01/05/2025 |
| --- | --- | --- | --- |

1. DESCRIPTION OF WORK

For the use of Battery Operated Drills

2. PERSONNEL COMPETENCE REQUIRMENTS

All Personnel to be fully trained.

3. SUPERVISORS DUTIES AND RESPONSIBILITIES

To asses the work to be carried out and ensure that all personnel required have been fully trained and carry the correct PPE.

4. MEANS OF ACCSESS

Access is required to all areas equipment is located. Some items my need to be fitted behind panels, voids or roof spaces. Step Ladders

5. MACHINERY, PLANT AND EQUIPMENT

Battery Operated Drill, Cable and Stud Detectors, Step Ladders

6. LIFTING OPERATIONS AND EQUIPMENT

N/A

7. MATERIALS STORAGE, SECURITY AND HANDLING

8. SEQUENCE OF OPERATIONS

- Ensure the area is closed off and ‘2 meter’ social distancing rule is possible - Ask the customer if they any Asbestos on site, if so ask for a copy of the Risk Assessment and plan of any area containing Asbestos prior to drilling. You MUST NOT drill if there is any Asbestos in the vicinity of the install area. - Visually check the drill and charger for any damage (DO NOT USE DAMAGED). If the charger is over 1 year old check for an in date PAT (portable appliance test) label (DO NOT USE IF OUT OF DATE). - Close off the area drilling is to take place, if this is not possible erect warning signs. - Ensure there is adequate lighting and the floor surface is free of obstacles, even and dry. Inspect the required drill bit to be used for damage (DO NOT USE A BLUNT OR DAMAGED DRILL BIT). - Insert the required drill bit into the drill in accordance with the manufacturer’s instructions. - Visually check the area to be drilled for loose plaster or tiles. Check the area to be drilled for hidden electricity cables, water/gas pipes and metal struts using a cable and stud detector (DO NOT DRILL IF THERE IS ANY INDICATION OF HIDDEN CABLES OR PIPES). - Tuck in loose clothing, tie back long hair and remove finger rings. Don eye protection and dust mask. - Commence drilling to the required depth. Once all holes have been drilled return the drill to its correct stowage. - Safely dispose of any waste materials and clean down surfaces of dust and debris. - Remove gloves, wash/sanitize hands

9.PERONAL PROTECTIVE EQUIPMENT

Safety Glasses, Safety Footwear, Dust Mask

10. PROTECTION OF THIRD PARTIES

Close of area and erect warnings signs

11.ENVIROMENTAL ISSUES

All waste is to be removed from site and disposed of in accordance with the Environmental Protection Act 1990 and Elite Services Holdings company policy.

12. FIRST AID AND EMERGENCIES

All incidents and accidents are to be reported to the customer in the first instance and then to Elite Healthcare. All Elite Services Holdings Ltd service personnel carry 1st aid kits on there vehicles. All engineers must carry mobile phones whilst working alone.

13. OTHER INSTRUCTIONS

Follow safe Working Systems for items being fitted.