| Safe System of Work |  |
| --- | --- |

| Company name | Elite Services Holdings Ltd | Department | Service |
| --- | --- | --- | --- |

| Contract | Customers Sites | Prepared By | Gareth Lewis |
| --- | --- | --- | --- |

| Date prepared | 01/05/2024 | Review date | 01/05/2025 |
| --- | --- | --- | --- |

1. DESCRIPTION OF WORK

The installation of Sensor Flush Watersavers

2. PERSONNEL COMPETENCE REQUIRMENTS

All Personnel to be fully trained.

3. SUPERVISORS DUTIES AND RESPONSIBILITIES

To assess the work to be carried out and ensure that all personnel required have been fully trained and carry the correct PPE.

4. MEANS OF ACCSESS

Access is required to all areas equipment is located. Some items my need to be fitted behind panels, voids or roof spaces. Step Ladders

5. MACHINERY, PLANT AND EQUIPMENT

Battery Operated Drill, Cable and Stud Detectors, Step Ladders, Pipe Cutter, Adjustable spanners, Freezer kit (if needed) Screw drivers, Braising kit (if needed) Note: A hot work permit must be obtained if any braising is to be carried out

6. LIFTING OPERATIONS AND EQUIPMENT

N/A

7. MATERIALS STORAGE, SECURITY AND HANDLING

8. SEQUENCE OF OPERATIONS

- Ensure the area is closed off and ‘2 meter’ social distancing rule is possible - Ask the customer if they any Asbestos on site, if so ask for a copy of the Risk Assessment and plan of any area containing Asbestos prior to drilling. You MUST NOT drill if there is any Asbestos in the vicinity of the install area. - Restrict Access to toilet area. - Remove any service panels if applicable. - Ensure the urinal system is empty and that it has been fully drained. - Ensure water supply has been isolated at the urinals service valve or mains stopcock, if this can’t be done a freezer kit may be used as per the installer’s discretion. - Cut the cistern feed pipe and fit the solenoid valve as per the manufactures instructions. - Turn on water supply and check for leaks - .Install sensor unit as per manufactures fitting instructions and program as applicable to the customers environment - Replace any removed panels if applicable. - Test and observe one full cycle of the sensor flush operation. - Remove all packaging and clean any debris left from install. - Reopen Toilet area - Remove gloves, wash/sanitize hands

9.PERONAL PROTECTIVE EQUIPMENT

Safety Glasses, Safety Footwear, Dust Mask, Braising gloves if applicable, KN95 protective mask

10. PROTECTION OF THIRD PARTIES

Close of area and erect warnings signs

11.ENVIROMENTAL ISSUES

All waste is to be removed from site and disposed of in accordance with the Environmental Protection Act 1990 and Elite Services Holdings company policy.

12. FIRST AID AND EMERGENCIES

All incidents and accidents are to be reported to the customer in the first instance and then to Elite Healthcare. All ‘Elite Services Holdings’ service personnel carry 1st aid kits on there vehicles. All engineers must carry mobile phones whilst working alone.

13. OTHER INSTRUCTIONS

Follow safe Working Systems for items being fitted.